Documentation

INTRODUCTION AND PROJECT OBJECTIVES

In modern high-performance motorsport engineering, the precision of race strategy forecasting directly dictates competition outcomes. Strategic calculation errors triggered by unmodeled tire degradation cliffs or incorrect weather crossover positioning lead to immediate track position losses on the pit lane and severe degradation of the final race time.

The primary objective of this project is to build a closed mathematical simulation framework capable of evaluating:

- Linear vehicle mass reduction due to constant fuel consumption .

- Non-linear degradation of rubber compounds under mechanical track stress and thermal conditions.

- Sudden, catastrophic lap time loss when tire structural limits are violated.

- Dynamic rain forecast adjustments altering track surface conditions.

This research paper presents the technical architecture, empirical calibration, and brute-force optimization loops developed for the "F1 Race Strategy Command Board" application. The software simulates lap-by-lap vehicle metrics and provides strategic engineers with precise real-time delta calculations to track tactical evolution versus initial opening forecasts.

# 1. PHYSICAL ENGINE MATHEMATICAL ARCHITECTURE

The simulator core evaluates vehicle dynamics by superimposing independent lap-time penalties. The total calculation for the final lap time (T) on any given lap index (n) is synthesized as a function of the car's baseline performance potential, track evolution progression, and five individual physical degradation variables:

Tlapn= Tbase⋅En+ Δtgripn+ Δtfueln+ Δtfailure+ Δtwing+ Δtpenalty

- T represents the simulated total lap time executed on the active lap index.

- T base is the absolute performance ceiling of a completely empty car on a baseline optimal track surface.

- E represents the linear track evolution coefficient modeling grip improvements as rubber is deposited on the asphalt.

- t wear is the mechanical pace degradation caused by the deterioration of the rubber tread.

- t fuel is the linear time reduction earned as the vehicle weight drops.

- t failure is the piecewise-defined front axle failure penalty applied when the tire carcass is compromised.

- t wing is the aerodynamic time penalty resulting from mechanical damage to the front wing elements.

- t penalty is the sporting time penalty served stationary by the driver under FIA race control orders.

## 1.1. Modeling Vehicle Weight Dynamics and Fuel Consumption Matrix

Vehicle overall mass decreases linearly with every completed lap. As fuel is consumed, the normal load applied to the tire contact patch drops, reducing kinetic energy requirements during acceleration and heavy braking phases. This mass reduction yields a consistent pace advantage evaluated at a 0.3-second improvement per 10 kg of consumed fuel weight.

The instantaneous fuel mass remaining (M) on any given lap index is governed by a minimum-boundary volume function:

Mfueln= max0.0, Mstart- n - 1⋅qfuel

The linear pace correction factor (t топлива) is computed via the total burned fuel mass delta and enters the master equation as a negative value, representing vehicle speed acceleration:

Δtfueln= Minitialtank- Mfueln10.0⋅Cfuel

Where:

- M represents the instantaneous fuel weight remaining in the cell.

- M start is the starting fuel allocation load initialized at 110.0 kg.

- n is the active lap order index.

- q is the track-specific fuel consumption rate.

- M tank is the physical tank capacity volume at session launch (110.0 kg).

- C is the fuel chassis sensitivity constant scaled at -0.3 seconds per 10 kg delta.

## 1.2. Differential Tire Wear Progression Model

The physical wear rate of the tread rubber layer per lap (W) is calculated as a multiplicative function derived from tire mileage age, asphalt abrasive stress, temperature windows, and competitive turbulent trailing wake interaction:

Wlapn= ΔWbase* 1.0 + Aage* Kaccel* Kstress* Ftemp* Faero* Fweatherwear

Where:

- W is the accumulated tire wear percentage evaluated over the current lap.

- W base is the baseline compound degradation constant per lap under optimal laboratory conditions (Soft: 1.80%, Medium: 1.20%, Hard: 0.80%).

- A is the current age of the tire set measured in completed laps.

- K accel is the rubber aging exponential acceleration modifier.

- K stress is the mechanical abrasion and surface roughness constant of the circuit.

- F temp is the track surface temperature scaling modifier.

- F aero is the dirty air scaling coefficient magnifying wear by 1.15 when running behind another car.

The residual effective tire health percentage (H) is derived from the difference between the prior state and the per-lap wear deduction:

Hn= max0.0, Hn-1- Wlapn

The simulator multiplies the current wear progression by a factor of 2.0 as soon as tire health drops past the critical tire performance cliff threshold (H < 30.0%).

## 1.3. Thermodynamic Window Modifiers

The molecular structural degradation rate of racing tire rubber is deeply linked to the track surface temperature. Excessive heat disrupts the polymer bonds within the compound matrix. The temperature scaling modifier coefficient follows a discrete piecewise-linear function:

F_temp = piecewise window threshold matrix (Dry / Damp / Wet temperature conditions)

{ 0.80, if T_track < 20.0°C

{ 0.90, if 20.0°C <= T_track < 25.0°C

F_temp =   { 1.00, if 25.0°C <= T_track <= 40.0°C

{ 1.15, if 40.0°C < T_track <= 45.0°C

{ 1.35, if 45.0°C < T_track <= 50.0°C

{ 1.60, if T_track > 50.0°

## 1.4. Non-Linear Power-Law Mechanical Grip Decay Function

The mechanical grip loss experienced as the active rubber layer wears thin follows a non-linear power-law curve. The net pace reduction (t grip) is computed as follows:

Δtgripn= 100.0 - Hn100.01.6⋅Cwear

Where:

- H represents the instantaneous residual tire health percentage.

- C wear is the mechanical pace loss constant specific to each allocated compound: Soft (3.5s), Medium (2.8s), Hard (2.2s).

## 1.5. Piecewise Bounded Carcass Failure and Puncture Overheads

When tire health drops past critical safety limits, structural failures occur. Delamination and tire blowout scenarios are simulated using a piecewise step-function penalty (t failure):

[ 0.0,  if H >= 30.0%

Δt_failure =   [ 2.5,  if 15.0% <= H < 30.0%

[ 15.0, if 0.0% < H < 15.0%

[ 30.0, if H = 0.0%

Applying a severe +30.0 second penalty at H = 0% accurately replicates a flat tire blowout where the vehicle is restricted to a limping pace. This mathematical penalty forces the strategic optimizer to completely penalize and reject strategy paths that attempt to overextend tire usage beyond physical structural safety limits.

# 2. CIRCUIT TELEMETRY DATA CALIBRATION AND CASE STUDIES

To ensure accurate engineering baselines, the model parameters were tuned against official FIA lap timing records using data parsed via the FastF1 telemetry library.

## Case 1: High Lateral Load and Banked Asphalt Abrasion (Zandvoort)

- Calibration Dataset: Max Verstappen's Hard tire stint telemetry at the Dutch Grand Prix, evaluated during defensive operations in clean air.

- Physical Analysis: The Zandvoort circuit features banking angles up to 18 degrees. These profiles combine massive centrifugal and vertical forces, over-loading the tire sidewalls and driving high internal rubber temperatures.

- Engineering Output: Data parsing isolated a pure mechanical pace loss slope of +0.082 seconds per lap for the Hard compound. This steep wear profile calibrated the circuit mechanical stress constant at a high value of 1.65 within the track specifications matrix.

## Case 2: Tread Block Flexing and Negative Wear Optimization (Istanbul Park)

- Calibration Dataset: Valtteri Bottas's Intermediate compound telemetry during the opening 35 laps of the Turkish Grand Prix on a damp, drying track.

- Physical Analysis: Grooved wet tires experience tread block flexing on dry track regions, generating severe thermal strain. However, as the tread wears down, the groove depth decreases and block stiffness increases, turning the worn tire into a functional slick profile. The contact patch with the asphalt expands, temporarily counteracting compound chemical grip loss.

- Engineering Output: The isolated telemetry revealed a net negative wear curve of -0.003 seconds per lap (the driver was gaining pace as tire mileage grew). This parabolic behavior was integrated into the simulator's Intermediate compound model: pace delta decreases between 100% and 40% tire life, followed by a severe +5.0s drop once the rubber is completely worn off.

## Case 3: Thermal Blistering Under Fluid Coolant Deficit (Imola)

# Calibration Dataset: Lewis Hamilton's Full Wet tyre opening stint (laps 1–9) at the Emilia Romagna Grand Prix, tracking the transition from Wet to Damp track states.

# Physical Analysis: Heavy wet tread compounds are designed to operate with water acting as a structural coolant to stabilize rubber core temperatures around 60°C. When a dry racing line forms, this coolant disappears, causing compound temperatures to spike beyond 130°C and inducing severe thermal blistering.

# Engineering Output: Telemetry tracking exposed an exponential performance drop: Full Wet tire wear scales by a factor of x2.5 on dry asphalt, generating a severe, compounding pace loss of +0.35 seconds per lap.

# 3. GLOBAL TAKTICAL STRATEGY SEARCH ENGINE ALGORITHM

The system computes optimal race strategies via the StrategyOptimizer module, which executes an ordered brute-force search over a bounded tactical matrix. The optimizer evaluates all permutations independently from Lap 1 to prevent any live driver state biases from corrupting the baseline forecast.

## 3.1. Bounded Search Space Filtering Rules

The optimization engine builds a matrix of legal strategy permutations. To protect CPU resources from freezing under multi-stop combinatorial explosions, 4 tactical filters are hardcoded into the plan generator:

- Search Step Window (lap step = 7): During active race operations, the algorithm steps through possible pit lap indices with a gap of 7 laps. This compresses the permutation matrix to a few dozen legal steps, preventing Streamlit screen stutters on button clicks. A finer resolution of lap_step = 3 is used exclusively on Lap 0 before race launch.

- Minimum Stint Window (gap = 6): Every consecutive pair of pit stops must maintain a hard spacing window of at least 6 laps to eliminate irrational back-to-back box calls.

- Sequential Compound Lock: The plan generator automatically skips plans that fit the exact same compound sequentially (e.g., Soft to Soft), unless an environmental weather crossover event occurred between those pit windows.

- Circuit Dynamic Depth Limits: Slow, low-wear street circuits (Monaco, Monza) restrict the plan generator strictly to 0-stop and 1-stop plans to protect computing overhead, while high-wear open circuits evaluate up to 2-stop chains.

## 3.2. Real-Time Strategic Evolution Tracking

At Lap 0, prior to any live lap execution, the system locks down the total race completion time of the absolute fastest strategy profile as our baseline benchmark:

Tbaseline= TtotalraceStrategy #1​Lap 0

During active race simulation, as the user drives laps and modifies car parameters, the optimizer continuously re-calculates the remaining distance. Each strategic card displays a real-time evolution delta (ΔT) evaluating current choices against the initial baseline:

ΔTevolution= Telapsedlive+ Tprojectedremaining- Tbaseline

Where:

- ΔT represents the net time difference of the total race relative to the Lap 0 prediction.

- T elapsed is the banked live race time already driven on track by the user.

- T projected is the estimated remaining race time computed for that specific strategic path.

- T baseline is the unmodifiable reference time captured on the grid prior to race start.

# Conclusion

The developed strategic simulator successfully bridges the gap between tire thermodynamic formulas, vehicle weight loss matrices, and discrete brute-force pathfinding algorithms. Calibrating the physical constants against real FIA Formula 1 telemetry ensures highly authentic race timelines. The framework provides instant strategic evolution calculations, coordinates inventory depletion securely, and stands as a complete, high-performance tactical analysis software platform.